# Measured-vs-Expected Flux from Standard Stars (RCALIBFRAC):

# Sky-Subtraction Bias, and Which Variable to Use

Status: for the Data Systems / calibration team (J. Guy). Part of the telemetry_mining / Exposure-module documentation family; companion to DAR_FIBER_LOSS_REPORT.md (the DAR study that produced the sky-subtraction finding), FIBER_LOSS_METRICS.md (the artifact-free L_see/L_field metrics), API.md, and FIELDS.md.

K. Honscheid (OSU) with Claude (Anthropic) · rewritten 2026-08-13 (supersedes the 2026-07-24 draft, which predated the sky-subtraction finding and the geometric metrics).

Purpose. For telemetry / observing-conditions studies we want a per-exposure variable that says “this is what we measured, this is what we should have measured” — a flux-loss / throughput diagnostic — so we can correlate it against wind, seeing, transparency, mirror temperature, positioning, and so on. Standard stars are the only clean on-sky “known truth.” This note pins down how the candidate variables are built (from the desispec source), what each one does and does not measure, and — the main addition since the first draft — why the obvious choice, RCALIBFRAC, is not a bias-free flux-loss estimator: beyond two construction quirks, it carries a zenith-tied dipole injected at the sky-subtraction step, which we tracked down but did not fully pin. It closes with what to use for which study, and two concrete items for desispec.

## Executive summary

- The truth reference is the standard star: measured r-band flux / model r-band flux = end-to-end throughput (atmosphere × telescope × instrument × fiber-aperture loss). RCALIBFRAC (the calibstars QA product) is the pipeline’s normalized version of this ratio.

- RCALIBFRAC carries three biases, in increasing order of subtlety:

  - Median normalization — each exposure’s median is forced to 1, so RCALIBFRAC is blind to absolute throughput and reports only within-exposure, fiber-to-fiber variation.

  - A mid-2025 aperture-correction step — a point-source (flat→psf) correction (PR #2484) was added in mid-2025; because daily is not reprocessed, this is a step discontinuity across the archive.

  - A sky-subtraction dipole (new) — RCALIBFRAC’s spatial structure includes a zenith-tied, airmass-growing dipole that looks like DAR but is a flux-processing bias entering at subtract_sky, general to all point-source spectrophotometry — not a RCALIBFRAC-specific effect. We localized it and ruled out several mechanisms, but the exact algorithmic step is not pinned.

- Which variable to use (see §5 for details. L_see/L_field are part of the telemetry_mining Exposure module):

  - Seeing-loss level (the clean stand-in for RCALIBFRAC’s seeing content) → L_see — artifact-free, GFA-based, and available now. This is what most level studies (dome / mirror seeing, transparency-free conditions) actually want.

  - The total measured loss, to attribute across all channels (transparency, positioning, throughput, …) → RATIO_RAW (raw absolute measured/model). Its dipole averages out over the focal plane, so it is clean for level work — but it does not exist yet (needs reprocessing), and its non-redundant content vs L_see is only the non-seeing channels. For the total throughput specifically the flux-cal scale is an already-on-disk alternative.

  - Spatial / DAR field structure → no flux ratio is clean. Use L_field (field-distortion; L_see is the level companion); the dither offset field is the only clean flux-independent geometric probe. We chose this GFA-based geometric approach for the telemetry_mining / Exposure-module product.

  - Depth / “good exposure?” → EFFTIME_SPEC / TSNR2 (convenient but conflates throughput, transparency, sky, and time).

- For desispec (details in the last section): one finding to hand over — the subtract_sky point-source dipole (a real, zenith-tied bias in a pipeline product; localized to that step, mechanism not yet identified). The known mid-2025 RCALIBFRAC discontinuity is not an ask on the team — for our purposes it could simply be avoided (if needed) by working from a uniformly-reprocessed release (e.g. Matterhorn or Nevis) instead of the daily archive.

## 1. The known truth: standard stars

A standard star has a model SED (MODELRFLUX etc.), obtained by fitting a stellar template to the star’s broadband photometry (imaging / Gaia). That model is “what we should have measured” above the atmosphere. The measured flux is what actually reaches the detector through the fiber. Their ratio is the end-to-end system throughput — atmosphere (extinction + transparency) × telescope × instrument × fiber aperture loss. Standard stars are the only per-exposure, on-sky, absolute reference we have, so every measured-vs-expected variable below is anchored on them. (Caveat: the “truth” is only as good as the template/photometry calibration — the best available reference, not a perfect absolute.)

## 2. How RCALIBFRAC is built — as we understand it

From desispec/scripts/select_calib_stars.py, per exposure:

- Read the r-camera frames, apply fiberflat, subtract sky.

- Apply a point-source (flat→psf) aperture correction to the flux (line 131–132): flux *= flat_to_psf_flux_correction(fibermap, exposure_seeing_fwhm=1.1, normalize=False) — see §2.1.

- ratio = rflux / MODELRFLUX per star (measured / model, summed 6000–7300 Å).

- medval = median(ratio) over the exposure’s stars (line 168) — the absolute throughput …

- … then RCALIBFRAC = ratio / medval (line 173) — medval is divided out and not saved.

- VALID flags 3σ RCALIBFRAC outliers and G−R color mismatches.

Layer 0 — median normalization. Step 5 forces each exposure’s median RCALIBFRAC to 1, so RCALIBFRAC carries only within-exposure, fiber-to-fiber variation — blind to the exposure’s absolute throughput (anything dimming all fibers together is normalized away). A per-exposure “mean loss = 1 − mean(RCALIBFRAC)” therefore measures the differential loss, not the absolute one.

Layer 1 — the aperture correction (step 2, added mid-2025). RCALIBFRAC measures the residual after the pipeline’s modeled static aperture loss has been divided out.

### 2.1 The flat→psf aperture correction (fiberfluxcorr.flat_to_psf_flux_correction)

Per fiber it computes the captured point-source light fraction and divides the flux by it:

- inputs: FIBER_X/Y → radial plate scale; DELTA_X/DELTA_Y → the fiber-to-target positioning offset; and a seeing FWHM → PSF size σ.

- fiber_frac = FastFiberAcceptance("POINT", σ, offset); correction = 1 / fiber_frac / platescale².

So the corrected flux ≈ F_true · fiber_frac_actual / fiber_frac_model, and after the ratio/median step RCALIBFRAC retains only the residual fiber_frac_actual / fiber_frac_model — i.e. what the model gets wrong or can’t see: seeing ≠ the assumed 1.1″; intra-exposure / dynamic effects (DELTA_X/Y is one online snapshot, so wind jitter, tracking, and DAR drift survive); optics/coordinate-model error; and a purely radial reshaping via 1/platescale².

### 2.2 The same correction lives in three products — with different seeing

| product | file | seeing used | notes |
| --- | --- | --- | --- |
| RCALIBFRAC (calibstars QA) | select_calib_stars.py | hardcoded 1.1″ | normalize=False; then median-normalized |
| flux-cal vector (science flux) | fluxcalibration.py | actual/estimated exposure seeing | vector = C_fiber / FLAT_TO_PSF_FLUX; wavelength-resolved |
| TSNR2 → EFFTIME | tsnr.py | (as used there) | folds the same correction |

The science flux calibration uses the actual seeing (a more correct aperture correction); the calibstars QA uses a fixed 1.1″ (a cruder version whose residual still carries a seeing-vs-1.1″ signal).

## 3. The mid-2025 discontinuity: RCALIBFRAC changed, and daily is not reprocessed

Data releases are reprocessed uniformly; daily is not — each night is processed once with the then-current code. So the mid-2025 addition of the aperture correction (PR #2484, “add psf correction to RCALIBFRAC”, merged 2025-05-08, released 0.70.0) is a step discontinuity in RCALIBFRAC across the daily archive at the deployment date, present at all airmass. This is the same shift the DAR fiber-loss study found independently as its “Finding 1” level change. This is a known pipeline change, not a defect; the practical handling is on our side — use a uniformly-reprocessed data release (e.g. Matterhorn or Nevis) rather than the daily archive, which removes the step by construction. (Any cross-time comparison that does use daily RCALIBFRAC must respect this boundary.)

## 4. The deeper bias: a sky-subtraction dipole in the measured flux

This is the finding that motivated the write-up, and the reason RCALIBFRAC is not a clean flux-loss estimator for spatial (DAR) studies. Full evidence, figures, and DAR context are in DAR_FIBER_LOSS_REPORT.md (Parts IV–V); summarized here because it is fundamentally a measured-flux property.

What it is. Decomposing RCALIBFRAC’s per-fiber loss map into multipoles in a zenith-aligned (derotated) frame yields, besides the real radial and quadrupole terms, a rotating dipole D_rot — a coherent, zenith-tied, airmass-growing whole-field gradient, ~2× the quadrupole (DAR). It looks like a DAR boresight offset. It is not.

It is not geometric. DESI’s dither sequences (Schlafly et al. 2024, arXiv:2403.05688) measure the actual fiber-to-light offset field by a completely different technique (a geometric centroid fit, not a flux ratio). Fit with the same decomposition (night-clustered over 12 nights), the real offset field shows no rotating dipole (flat ~0.04″, ~5× below the loss-implied amplitude, slope vs tan z straddling zero) while its quadrupole grows and tracks Q_rot on the same data — a built-in positive control. There is no coherent geometric offset; starlight is not centroiding off the fiber in a zenith-tied way.

It is in the measured flux, not the model or the aperture correction. Reconstructing RCALIBFRAC from its ingredients and fitting the dipole separately to the measured-flux and model-flux terms puts essentially all of it in the measured flux (data-term D_rot 0.041→0.094 with airmass vs model-term 0.013–0.019, flat). The aperture correction is excluded twice: the dipole is at full strength in an epoch before the correction existed (0.0341 at am>1.4), and a direct fit to the correction factor is 50–100× too small.

It enters at subtract_sky. Snapshotting the measured flux at each construction stage (raw extraction → +fiberflat → +sky subtraction → +aperture correction) and refitting: fiberflat removes the fixed instrument-frame dipole; the rotating dipole roughly doubles, and gains its airmass growth, specifically at sky subtraction; the aperture correction changes nothing after. (A smaller rotating dipole is already present in the raw extracted counts — see Limitations — but sky subtraction is where most of the amplitude appears.)

Its character — three discriminators (all on the sky-subtraction-stage flux): - Star brightness: D_rot increases with brightness, faster than flux — the opposite of the ∝1/flux scaling a simple additive sky-background residual would give. Rules out the naive additive-residual picture. - Sky level: D_rot scales strongly with the pipeline’s per-exposure sky brightness (dark sky → consistent with zero; bright sky → full amplitude). The effect needs a real sky background to act on. - Sky-fiber residual: (data − sky model) measured directly at OBJTYPE=='SKY' fibers is null (~350–800× below D_rot, flat). The sky model is not spatially wrong; the effect requires the star’s own signal to be present.

What we ruled out inside subtract_sky. Three named mechanisms each came back null — _model_variance ivar inflation (uses the mean sky, not the fiber’s own flux, so cannot couple to star brightness), the sky-line throughput correction (diluted in the broad band), and — decisively — a bisector test that kills the entire class of inverse-variance-weighting artifacts in one shot (recomputing the flux as an unweighted mean leaves D_rot unchanged). So it is in the flux values themselves, not the weighting.

It is general to point-source spectrophotometry, not RCALIBFRAC-specific. The strongest generality check builds the metric a calibration expert would use — spectrograph counts/s vs external Legacy photometry, over all point sources (MORPHTYPE=='PSF') — which shares nothing with RCALIBFRAC except the sky-subtracted flux (no stellar model, no flux calibration, no standard-star selection; ~470–600k stars). The same dipole (and quadrupole) survive and grow with airmass, D_rot/Q_rot ≈ 1.5–1.7:

| am cut | n point sources | D_rot | Q_rot | D_rot/Q_rot |
| --- | --- | --- | --- | --- |
| > 1.2 | 596,155 | 0.0515 | 0.0341 | 1.51 |
| > 1.4 | 470,451 | 0.0661 | 0.0396 | 1.67 |
| > 1.8 | 158,170 | 0.1019 | 0.0589 | 1.73 |

So there is no clean counts-based DAR metric until the coupling is understood; the geometry (dither, and the GFA-based metrics) is the only artifact-free route.

What we did not pin: This is a strong lead, not a confirmed diagnosis: - The stage attribution (“subtract_sky specifically” vs an adjacent step such as extraction) rests on an external reimplementation validated against production only at the final RCALIBFRAC value, not at each intermediate stage (desispec does not save those). “It is in the sky-subtracted flux” is production-validated (the point-source metric runs on production flux); “it enters at subtract_sky” is the lead to verify in the real code path. We ruled out several mechanisms and the ivar-weighting class; the true cause could be one we did not test, or in an adjacent step. - A real (non-bug) alternative — a field-scale sky-brightness gradient the mean-sky model misses — is disfavored by the sky-fiber null but not formally excluded. Differential atmospheric extinction is a legitimate physical candidate but ~10× too small and should appear already in the raw counts.

Bottom line for this note: the bias is a real, zenith-tied, airmass-growing property of the sky-subtracted point-source flux; it contaminates RCALIBFRAC’s dipole entirely (there is no real DAR dipole) and its quadrupole partially (a real DAR quadrupole, ~11 μm at high airmass, survives at dark sky). No choice of flux variable removes it — it is upstream of the ratio, the normalization, and the aperture correction.

## 5. The candidate variables, and which to use

| variable | construction | what it folds | best for | availability |
| --- | --- | --- | --- | --- |
| RCALIBFRAC | measured/model, aperture-corrected (fixed 1.1″), median-normalized | within-exposure differential loss; spatial structure carries the sky-sub dipole | instrument-fixed spatial patterns (per-petal, positioner); not DAR/zenith-tied structure | now (calibstars) |
| RATIO_RAW (proposed) | measured/model, no aperture correction, un-normalized | the total measured loss — all channels: transparency, extinction, instrument throughput, and aperture loss | attributing the total loss, esp. the non-seeing channels (transparency, positioning); dipole averages out, so ok for level | needs reprocessing |
| medval / MEDVAL_RAW | per-exposure median of the ratio | absolute r-band throughput scalar | absolute throughput per exposure | medval computed but not saved → reprocessing |
| flux-cal scale | r-band normalization of the per-exposure fluxcalib-*.fits vector | absolute throughput after aperture correction (actual seeing), wavelength-resolved | absolute, science-grade throughput | vector on disk (redux tree); scale derived — accessor not built |
| EFFTIME_SPEC / TSNR2 | achieved depth | throughput × transparency × sky × time (conflated) | exposure quality / depth flag | now (redux_row, petalqa, fiberqa) |
| L_see (level) / L_field (spatial) | GFA imaging (seeing / intra-exposure guide drift) → acceptance model | L_see: the seeing-loss level (a scalar); L_field: DAR field-distortion — both artifact-free, dipole-free | L_see: seeing-loss level (the clean, available stand-in for RCALIBFRAC’s seeing content); L_field: DAR / spatial structure | now (Exposure); see FIBER_LOSS_METRICS.md |

What to use when. The key distinction, easy to blur: L_see is a modeled predictor of the seeing channel; RATIO_RAW is the total measured outcome you attribute. They sit on opposite sides of the regression — not substitutes. (A cloudy but good-seeing exposure makes this concrete: L_see reads “low loss,” RATIO_RAW reads “high loss” — the clouds are in RATIO_RAW and invisible to L_see.)

- Seeing-loss level (dome / mirror seeing, transparency-free conditions) → L_see — the artifact-free, GFA-based, available-now measure. It is essentially a deterministic function of the GFA seeing, so it is the clean stand-in for RCALIBFRAC’s seeing content and a key predictor — but blind to everything that isn’t seeing (transparency, positioning / DAR offset, instrument throughput). For most level studies this is what you actually want, and it needs no reprocessing.

- The total measured loss, to attribute across channels → RATIO_RAW is the right dependent variable — measured/model keeps the full throughput (transparency, extinction, instrument) and every aperture-loss channel, and its sky-sub dipole averages out over the focal plane (a pattern, monopole-null; clean for level by averaging, not by being artifact-free). Two caveats: it does not exist yet (§6), and its only content beyond L_see is the non-seeing channels — so it earns its reprocessing only if those are the target. (Interim, focal-plane-averaged RCALIBFRAC gives the differential level only — median-normalized, and mind the §3 epoch step.)

- Absolute, science-grade throughput → the flux-cal scale — the r-band normalization of the per-exposure flux-calibration vector, $DESI_SPECTRO_REDUX/<release>/exposures/<night>/<expid:08d>/fluxcalib-<cam>-<expid:08d>.fits (actual-seeing correction, wavelength-resolved). The product is on disk in the redux tree, but extracting the scalar scale needs a small accessor we have not built yet (the Exposure module has cframe/exposure_qa/calibstars path helpers, no fluxcalib).

- Spatial / DAR field structure → do not use a flux ratio. Use L_field (field-distortion), built from GFA imaging and never touching sky-subtracted flux; the dither offset field is the only clean flux-independent geometric probe. See FIBER_LOSS_METRICS.md and DAR_FIBER_LOSS_REPORT.md.

- Depth → EFFTIME_SPEC / TSNR2 (convenient but conflated — not a clean measured/expected).

Reconstruction note. With RCALIBFRAC and medval you recover the absolute per-fiber ratio (RCALIBFRAC × medval), but still the aperture-corrected (fixed-1.1″) value. The raw per-fiber map additionally needs dividing out psf_correction_i (recomputable from FIBER_X/Y, DELTA_X/Y) — i.e. the reprocessing path, which produces RATIO_RAW directly. None of these operations removes the sky-sub dipole (§4).

## 6. The clean fix for the level-attribution variable: reprocessing

The discontinuity is best sidestepped simply by working from a uniformly-reprocessed release (e.g. Matterhorn or Nevis) rather than the daily archive (§3) — no reprocessing needed. What a release still does not provide is an absolute, un-normalized variable (its RCALIBFRAC is still median-normalized and aperture-corrected). If that becomes necessary for the telemetry work, the option is to reprocess calibstars ourselves against a frozen release and save more information. Whether it is necessary is worth weighing first: L_see already covers the seeing-loss level (available), the flux-cal scale covers the absolute throughput, and L_field covers the spatial DAR — so RATIO_RAW earns its reprocessing only if the non-seeing aperture-loss channels (positioning, transparency-coupled losses) are the thing to attribute. Proposed enriched table:

| column | meaning |
| --- | --- |
| RATIO_RAW | measured/model, no aperture correction, un-normalized — the raw absolute per-fiber “truth” |
| RCALIBFRAC | current definition — continuity |
| MEDVAL_RAW, MEDVAL_CORR | per-exposure medians — MEDVAL_RAW is the absolute throughput; MEDVAL_CORR is the normalization constant (the two differ because the correction shifts the median) |
| N_STARS, MEDVAL_RMS | stars entering each median and its scatter — weight/quality-cut without reopening per-fiber data |
| PSF_CORRECTION | per-fiber correction factor — move freely between raw ↔ corrected |
| FIBER_X/Y, DELTA_X/Y | correction inputs, and the positioning offset (a regressor in its own right) |
| SEEING, MODELRFLUX, EBV, colors, VALID | context |

Save both MEDVALs explicitly (not recomputed): medval is the median over the pipeline’s RCALIBFRAC > 0 set before the VALID cut, so a later RATIO_RAW.median() over a differently-filtered set would not reproduce the exact normalization constant. Saving the scalar pins it and hands you the absolute per-exposure throughput with no per-fiber groupby.

Status: this suggested table has not been built — it remains a recommendation. The one-off reprocessing done for the sky-subtraction investigation (the stage-by-stage flux-chain decomposition and the point-source-photometry metric on production flux; see DAR_FIBER_LOSS_REPORT.md, Appendix B) established the finding but did not produce a reusable RATIO_RAW/MEDVAL calibstars product. So there is currently no clean, time-consistent, absolute measured-vs-expected variable in hand for level-attribution studies — building it is the concrete Data-Systems ask (§7, item 2).

## 7. For the desispec team

The one thing to hand over — the sky-subtraction point-source dipole. A zenith-tied, airmass-growing bias enters point-source spectrophotometry at sky subtraction — inherited by RCALIBFRAC, by a direct counts-vs-photometry metric, and by anything built on sky-subtracted point-source flux (the standard photometry-comparison metric is not an escape from it). It is a real bias in a pipeline product, and identifying the mechanism is a desispec question we cannot settle from outside. The concrete facts as a starting point: - Real, zenith-tied, airmass-growing bias in the r-band flux, scaling super-linearly with star brightness (neither simple additive nor simple multiplicative), entering at subtract_sky (present after, absent/smaller before, unchanged by the aperture correction after). - Requires the star’s own flux to be present (null at pure sky fibers) and scales with both star brightness and overall sky brightness. - Not produced by _model_variance ivar inflation, the sky-line throughput correction, or ivar-weighting of the reduction in general. - The exact algorithmic step is the open piece; the subtract_sky-vs-adjacent-step attribution should be verified in the real code path (it rests on our external reimplementation). Bright-vs-faint and sky-level splits are the signature to reproduce internally.

Not a request — a note on RCALIBFRAC continuity. For completeness, and explicitly not an ask on the team: the mid-2025 flat→psf aperture correction (PR #2484) is a well-known step in RCALIBFRAC across the daily archive, and the median normalization discards absolute throughput. Both are handled on our side — for telemetry / level work we work from a uniformly-reprocessed release (e.g. Matterhorn or Nevis) rather than the daily archive, which removes the discontinuity by construction, and if a truly absolute variable is ever needed we can reprocess calibstars against a frozen release ourselves to save the enriched table (§6). No pipeline change is being requested for this.

## Appendix — source references

Pipeline construction: - desispec/scripts/select_calib_stars.py — RCALIBFRAC (aperture correction line 131–132; medval line 168; normalization line 173; VALID cut). - desispec/fiberfluxcorr.py — flat_to_psf_flux_correction / psf_to_fiber_flux_correction. - desispec/fluxcalibration.py — flux-cal vector = C_fiber / FLAT_TO_PSF_FLUX (actual-seeing correction). - desispec/tsnr.py — TSNR2/EFFTIME also apply the correction. - desispec PR #2484 “add psf correction to RCALIBFRAC” (merged 2025-05-08, released 0.70.0).

Sky-subtraction investigation (full detail, figures, and DAR framing in DAR_FIBER_LOSS_REPORT.md, Parts IV–V and Appendix B): - analysis/dar_dipole/fit_dipole_quadrupole.py, affine_fit_lib.py — the multipole decomposition. - dither_offset_field_test.py — the geometric (dither) null; rebuild_rcalibfrac.py — model-vs-data split; flat_to_psf_dipole_test.py — aperture-correction exclusion; flux_chain_decomposition.py — the stage localization; sky_residual_test.py, variance_inflation_test.py, throughput_correction_test.py, unweighted_reduction_test.py — the §4 discriminators and subtract_sky mechanism tests; the point-source metric is the same decomposition on log(counts/s) − log(FLUX_R) for all MORPHTYPE=='PSF' fibers.