# Recommended DESI Fiber-Loss Metrics and Their Scope

K. Honscheid (OSU) with Claude (Anthropic) · 2026-08-06 (Exposure API added 2026-08-07; worked examples added 2026-08-10)

Companion note to DAR_FIBER_LOSS_REPORT.md. That report established that RCALIBFRAC — the standard-star measured/model flux ratio — is not a clean fiber-loss estimator: its sky-subtraction step imprints a spurious, zenith-tied dipole that is general to point-source spectrophotometry, not specific to the metric. This note records the practical consequence: which quantities are clean, what each can and cannot see, and how to build a normalized per-exposure metric from the guide cameras that behaves conceptually like RCALIBFRAC without inheriting the artifact.

## 0. Primer — what these metrics are, in plain terms

(A plain-language on-ramp; the detailed construction and scope follow in §1–§3.)

The principle: measure the guide-camera images, not the flux. The sky-subtraction artifact lives in the sky-subtracted spectroscopic flux, so any flux ratio inherits it. The escape is to use what the guide cameras (GFAs) photograph — guide-star images — which never touch sky-subtracted spectra. The GFAs give two kinds of imaging measurement, and both are artifact-free: how big the star images are (the PSF width, i.e. seeing) and where they are (centroids, and how they drift across the field). “Geometry” is shorthand for “guide-camera imaging” — and image size counts just as much as image position.

Two physical ways a fiber loses light. A star’s light misses the fiber for two independent reasons, both captured by the desimodel fiber-acceptance function A(σ, δ) — the fraction of a point source’s light that couples into the fiber given PSF size σ and star-to-fiber offset δ:

- Fat PSF (σ, seeing): even a perfectly centred star spills light past the fiber edge if the seeing disk is larger than the fiber. This is a whole-array effect (seeing is ~uniform across the field) → Metric A = L_see.

- Off-centre star (δ, offset): a displaced star loses more light. The offset is not uniform — DAR bends the field, so the edge sees a different offset than the centre. This is a spatial pattern → Metric B = L_field.

The two are named for use as L_see (the whole-array loss level set by seeing) and L_field (the loss that varies across the focal-plane field). Both are losses (L_); the suffix names what drives them. L_field is left deliberately general — it is sensitive to any low-order field distortion, DAR being the dominant one but not the only possible one (e.g. a rotator or hexapod anomaly).

Why Metric B is “scale + shear.” The field-varying offset is written δ(r) = G·r, with G a small 2×2 map measured from how the guide stars drift across the field during the exposure. Any such map splits into translation + rotation + scale + shear. Translation is removed by the guide loop and rotation by the hexapod, so neither reaches the fibers; what remains is scale (isotropic expand/shrink) and shear (stretch one axis, squeeze the perpendicular). DAR compresses the field along the zenith — a 1-D squeeze — which is exactly scale + shear together. Both are needed: shear alone yields a perfectly circular (radial) loss, and only the scale×shear cross-term produces the zenith-tied two-lobed (quadrupole) pattern that real DAR makes.

What goes in, and from where (data sources verified in the build scripts):

| metric | what it captures | shape | GFA quantity | data source |
| --- | --- | --- | --- | --- |
| A — L_see | seeing / aperture loss (the level) | scalar per exposure | PSF width (seeing FWHM) | offline GFA reconstruction (FWHM_ASEC from the GFA summary) — the gold-standard seeing |
| B — L_field (scale+shear) | DAR offset pattern | per-fiber map | guide-star drift across the field | ETC per-frame JSON (thru/dx_gfa,dy_gfa) |

Both are guide-camera imaging; neither uses sky-subtracted flux — which is what carries the artifact. The two draw on different reductions of the same guide data, chosen fit-for-purpose rather than as a quality compromise. Seeing exists in three reductions of increasing quality — real-time GFA, processed ETC, and the offline reconstruction (the gold standard) — and Metric A, which needs only one seeing value per exposure, uses the offline reconstruction (FWHM_ASEC). Metric B instead needs the per-frame guide-star-offset time series to measure intra-exposure drift; the ETC per-frame JSON (thru/dx_gfa,dy_gfa) provides that directly, whereas the offline reconstruction we have is a per-exposure summary and carries no intra-exposure information.

## 1. The principle: measure the geometry, not a flux ratio

The sky-subtraction artifact lives in the sky-subtracted spectroscopic flux, so any flux-ratio built on it (RCALIBFRAC, counts/s-vs-photometry, self-calibration) inherits it. The escape is to use geometric / imaging measurements — positions and PSF sizes from the guide cameras (GFAs) and the dither technique — which never touch sky-subtracted spectroscopic flux. A guide-star centroid or a PSF FWHM is immune to a bias that scales the flux level; that is why the dither offset field and the guide-star residuals both showed the real DAR quadrupole and no dipole, while every flux metric showed the dipole.

## 2. The three clean probes and their scope

| probe | what it measures | cadence | scope | good for | blind to |
| --- | --- | --- | --- | --- | --- |
| Dither offset field | true per-fiber fiber-to-light offset (peak of flux-vs-commanded-position) | sparse (special sequences, ~12 nights in our sample) | complete per-fiber field | ground truth; calibrating the others; any pattern search | routine per-exposure monitoring |
| GFA guide-star residuals (position) | affine offset field from ≤2 stars × 6 edge GFAs | every exposure | field-differential affine (rotation/scale/shear → through the quadrupole; the raw translation t is guide-loop-corrected, not delivered — see §3.1) | the low-order DAR pattern (G·r) | per-fiber / high-order structure; whole-array shift (needs delivered-offset telemetry, not raw t) |
| GFA transparency / FRACFLUX / seeing (throughput/PSF) | atmospheric transparency and the point-source fiber fraction from the GFA PSF | every exposure | global (edge-sampled seeing ≈ field seeing) | whole-array throughput influences (dome seeing, transparency) | field-dependent structure |

Two mechanism classes to keep straight: position/shift effects (temperature, wind, pointing) move the centroid and register on the residual channel; size/throughput effects (dome seeing, transparency) change coupling without moving the centroid and register on the FRACFLUX/seeing channel. A pure offset metric is largely blind to a symmetric PSF broadening, so the two channels are complementary, not redundant.

The scope wall is at affine, not at “global.” Six edge GFAs pin down a translation plus rotation/scale/shear — the shear is the low-order field-differential term, which is exactly why the guide-star residuals reproduced Q_rot. What the edge sampling cannot resolve is anything beyond affine: per-fiber positioner error, per-petal patterns, high-order multipoles. Field-dependent pattern searches therefore remain dither-limited until the subtract_sky coupling is fixed (which would make a clean per-fiber flux metric possible).

## 3. A GFA fiber-loss metric — the RCALIBFRAC analog

Both RCALIBFRAC and this metric estimate the same physical quantity: the aperture-coupling loss, the fraction of a point source’s light that misses the fiber, a deterministic function of the PSF size σ and the star-to-fiber offset δ through the fiber-acceptance function A(σ, δ) (desimodel FastFiberAcceptance, “POINT”). RCALIBFRAC estimates it empirically from the spectra (and inherits the artifact); this metric predicts it from imaging-measured σ and δ (artifact-free). It has two pieces with different inputs and scopes.

### What the metric produces, per exposure — three things kept separate

It outputs two artifact-free quantities; a third item that keeps getting folded in is a validation, not an output. Keep them distinct:

| # | name | what it is | shape | driven by | use |
| --- | --- | --- | --- | --- | --- |
| A | L_see — seeing level, 1 − A(σ,0)/A_ref | acceptance loss at perfect centering (δ=0) | scalar per exposure | GFA seeing σ only | the RCALIBFRAC-level replacement — the y-axis for external-influence studies (mirror ΔT, wind, …) |
| B | L_field — DAR field pattern, 1 − A(σ,\¦G·r\¦)/A_ref | acceptance loss across the field from the delivered offset | per-fiber map (radial gradient + quadrupole; dipole ≡ 0) | field-differential drift G = scale+shear | the DAR distortion pattern; field pattern searches |
| — | Drift/shear check | drift amplitude (arcsec) vs DAR edge offset ΔG; B’s quadrupole vs dark-sky Q_rot | validation, not an output | — | confirms A and B; does not feed examples |

The two “radial” things are not the same: A is a whole-array constant (a monopole level), while B has a radial gradient that grows toward the field edge. External-influence examples use A (the scalar); they do not average B’s grid, and they do not use RCALIBFRAC. dar_gfa_fiber_loss_metric_v2 is B (radial-only until the scale+shear fix); it is not A.

Design correction (from a first prototype that failed the validation gate). The two lessons below are baked into the construction: (1) use only the field-differential offset G·r, never the raw translation t; (2) validate in offset/arcsec space, not by round-tripping through the acceptance model.

### 3.1 The DAR spatial pattern — the field-differential offset, in arcsec

- Input: the field-differential offset field δ(r) = G·r, where G is the intra-exposure shear drift — the shear accumulated over the exposure (from the ETC per-frame guide-star offsets), not a single-epoch snapshot. The loss is an intra-exposure-accumulated effect (fibers placed once, star drifts over the exposure), so the drift is the physically-apt quantity; a static PMGSTARS/PMGWCS shear snapshot is a different, smaller quantity (in the first prototype the static shear gave ~7 μm at am~2 vs the loss-based ~11–15 μm — the static-vs-drift gap). [Validated at am ≲ 1.6 — see §3.3. At am > 1.8 metric B’s predicted quadrupole runs ~40% above the dark-sky flux for reasons not yet established (§3.3); treat it as approximate there.]

- G must be the full scale + shear compression, not shear alone. The DAR field-differential is a 1-D zenith compression δ(r)=c·(r·ẑ)ẑ, which decomposes into equal parts isotropic scale and shear (G = (c/2)I + (c/2)·shear). A pure-shear offset gives |δ|² = (e1²+e2²)(u²+v²) — perfectly radial, so its loss carries zero quadrupole (NERSC, 2026-08-06, verified algebraically); the loss quadrupole is the scale×shear cross-term, which needs both. So feed the scale drift s(t) (already in Test 4’s 6-param affine fit) alongside the shear drift. Note this does not affect the offset-space validation below: for a 1-D compression scale=shear, so the shear amplitude remains a valid proxy for ΔG.

- Use only G·r; do NOT include the raw translation t. The uniform (boresight) part of the guide-model residual is guide-loop-corrected before the science exposure begins (Part IV of the report; the dither shows no delivered uniform offset), so feeding raw t in reintroduces an offset the fibers never feel — in the first prototype it produced a large spurious dipole (D_rot/Q_rot up to 5.3). Only the field-differential term is delivered, and with it D_rot = 0 by construction, correctly.

- Validate in offset (arcsec) space, not loss space. Compare G directly to the DAR edge offset ΔG (0.11 / 0.16 / 0.22″ at am 1.48 / 1.69 / 1.86) — the comparison the guide-star cross-check already passed (0.102″ / 0.168″ vs 0.11″ / 0.16″). Do not round-trip G → A(σ,δ) → refit a loss-quadrupole → σ_eff·√(2Q) to reproduce a quantity you can check geometrically; that round-trip introduced a ~2× normalization and an airmass-shape distortion in the first prototype (an σ-consistency issue between the FF and the ΔG conversion).

- Convert to a loss only if a loss number is wanted: apply the acceptance model once, loss(r) = 1 − A(σ, |G·r|)/A(σ, 0), keeping σ consistent throughout.

### 3.2 The whole-array level — the σ / seeing channel (the genuinely new, t-free part)

For external-influence studies (dome seeing, temperature, wind) the whole-array loss level is what you want, and the cleanest channel avoids the offset entirely: - Dome seeing / PSF size: A(σ_seeing, 0) directly captures the seeing-driven whole-array fiber loss — no offset needed. Normalize to a nominal A(σ_nom, 0) for a fraction; regress against seeing / temperature telemetry. This is the clean, t-free metric for the dome-seeing / transparency use case. - Wind / thermal shift: needs the real delivered whole-array offset — from guiding telemetry / the applied mount corrections — not the raw guide-model t (which is guide-corrected). If a validated delivered-boresight quantity isn’t available, this channel stays open; the σ channel above covers the dome-seeing case regardless.

Scope: whole-array level (σ channel) + low-order field pattern through the quadrupole (G·r). By design it carries no delivered uniform dipole, and it is blind to per-fiber / high-order structure (§2).

### 3.3 Offset space vs loss space, and the high-airmass discrepancy

Two spaces. A field distortion can be expressed either as a physical displacement — offset space, in μm, how far a target sits from its fiber — or as the fractional light loss that displacement causes — loss space, obtained by pushing the displacement through the fiber-acceptance function A(σ, δ). The acceptance function is what links the two, and the metric can be built or checked in either. The measurement chain is indirect, and worth stating precisely: guide stars have no fibers. They drift from their nominal positions; from the ≤ 6 edge GFAs we fit a field distortion G; we then infer that a science fiber at position r has its target offset by G·r; and only then does the (fiber) acceptance function apply. So a loss-space number stacks an acceptance-inversion on top of an already edge-extrapolated, inferred fiber offset — several steps removed from a direct measurement, which is why it is treated with caution.

Validation. L_field (the scale+shear DAR quadrupole) was compared to the dark-sky RCALIBFRAC Q_rot → ΔG target (the artifact-minimized flux measurement), using the identical σ_eff·√(2Q) conversion for both:

| am cut | L_field predicted ΔG | dark-sky measured ΔG |  |
| --- | --- | --- | --- |
| > 1.4 | 7.5 μm | 8.7 μm | agree |
| > 1.6 | 10.9 μm | 9.9 μm | agree |
| > 1.8 | 16.3 μm | 11.1 [10.3, 12.1] μm | predicted ~40% high |

Because the conversion is identical, this is not a conversion/route artifact: at high airmass the geometry predicts more quadrupole loss than the flux measures.

The overshoot was diagnosed exhaustively (both sessions; Mac reproduced the numbers). Ruled out as the cause:

- outlier tail — only 2 of 176 exposures exceed am 2.05; excluding them gives 15.4 μm (barely moves);

- high-leverage exposures — jackknife shows a broad population effect, no single driver;

- target measurement uncertainty — the dark-sky target’s own bootstrap CI is tight, [10.3, 12.1] μm;

- small-sample bias — subsampling a fixed low-airmass population down to n = 90 does not inflate the median (only widens the scatter), so it is an airmass effect, not a sample-size effect;

- magnitude / noise rectification — L_field’s airmass growth matches the coherent (mean-first) drift statistic (ratio ≈ 4.2), not the magnitude-first (≈ 2.7) or RMS (≈ 2.3), so it is the genuine coherent quadrupole, not a rectified/noise-inflated one;

- scale contamination — the scale/shear drift ratio is flat at ~0.73 across airmass.

Physically real, and bracketed — the dither result. The one measurement that can adjudicate “geometry over-predicts” vs “flux suppressed” is the dither, which directly measures the delivered per-fiber offset (ground truth, artifact-immune). Its coherent zenith-frame quadrupole steepens with airmass ~as the physical am² DAR law — the delivered-offset shear grows ×2.5 and the cross-term quadrupole ×11 over am 1.15 → 2.06, against the am² expectations of ×3.2 (offset) and ×10 (squared). So the high-airmass steepening is physically real, not a metric artifact. Against that ground-truth am² law, both estimates deviate — in opposite directions: over a comparable span am² predicts a Q-ratio ≈ 2.3, the flux grows shallower (×1.6 → it under-represents at high am) while L_field grows steeper (×4.7 → it over-represents). So the true am > 1.8 quadrupole most likely sits between the flux’s 11 μm and L_field’s 16 μm, not cleanly on either. (Caveats: the dither is a static total offset vs L_field’s intra-exposure drift, and thin at high airmass (n ≈ 91) — so it brackets the value rather than pinning it.)

Everything simpler was ruled out first: two mechanism guesses (small-n rectification; RMS/undirected rectification) were explicitly tested and falsified (recorded so they are not re-proposed), and the tempting coincidences — the shear-drift amplitude (~11 μm) matching the flux, the loss-space value (~16) matching the undirected RMS drift — are most likely coincidental (the shear amplitude is a different quantity from a quadrupole ΔG; the RMS match fails on airmass scaling).

Practical guidance. L_field validates against the clean flux at am ≲ 1.6. For detecting the field pattern, tracking its relative variation, or anomaly searches, it is reliable at all airmass — the dither confirms the airmass behaviour is physical. The only soft spot is the absolute magnitude at am > 1.8, which is bracketed 11–16 μm (~±40%) rather than a single number. This caveat is specific to L_field’s field-pattern quadrupole; it does not affect L_see or any whole-array-level use.

### 3.4 Both metrics are free of the rotating dipole — by construction and in data

Escaping the RCALIBFRAC rotating dipole (D_rot) is the whole reason these metrics exist, so it is worth confirming they are free of it — not just structurally but against data.

- By construction. L_see is a whole-array scalar; it has no spatial structure, so it cannot carry a dipole (or any rotating pattern) at all. L_field uses only the field-differential offset G·r and drops the uniform translation t — the one term of an affine field that produces a dipole — so its D_rot is identically zero (verified: D_rot = 0.0000 at every airmass cut).

- In data. The rotating dipole is a large, highly significant feature of the flux: fitting D_rot to the RCALIBFRAC loss field gives a ≈ 77σ detection (n = 24,630 exposures). It is absent from the delivered geometry — the dither offset field (the ground-truth delivered offset, measured independently of the flux) shows the real DAR quadrupole but no rotating dipole (the dipole-null established in DAR_FIBER_LOSS_REPORT.md using the artifact-free xfiboff+xtel quantity). So the dipole lives in the sky-subtracted flux, not in the geometry these metrics are built from. That is the empirical basis for dropping t, and the confirmation that neither L_see nor L_field inherits the artifact they were built to avoid.

## 4. Calibrating and bounding it with the dither

On dither-overlap nights the dither gives the true per-fiber offset δ_true → true FF_true = A(σ, δ_true). Comparing the GFA-predicted (affine, edge-extrapolated) FF to it: - validates the affine extrapolation from the edge GFAs to the science field; - calibrates any bias/scale between predicted and true loss; - the residual FF_true − FF_GFA quantifies the per-fiber part the GFA cannot see — i.e. the metric measures its own scope boundary rather than asserting it.

Bonus cross-check: differencing this artifact-free loss against RCALIBFRAC on the same exposures isolates the non-geometric part — a direct handle on the sky-subtraction artifact (and, in principle, a way to de-artifact RCALIBFRAC for the low-order terms).

## 5. Implementation notes

- Data (NERSC-side): guide-star PSF/seeing per exposure (GFA reduction / ETC products); the affine offset field from the per-exposure PlateMaker product pm-<expid>.fits (PMGSTARS/PMGWCS residuals — the same source used for the Q_rot guide-star cross-check); the acceptance model from desimodel FastFiberAcceptance + load_platescale (σ, δ in μm via the platescale).

- Reference scales: σ_eff ≈ 52 μm was the effective acceptance scale found in the DAR study (fiber-size dominated at DESI seeing); use the real FastFiberAcceptance curve rather than a fixed σ_eff for the prediction.

- Validation target: the dither offset field (analysis/dar_dipole/dither_offset_field_test.py machinery) on overlap nights; and Q_rot/ΔG from DAR_FIBER_LOSS_REPORT.md as the known low-order pattern the demeaned-mode metric must reproduce.

## 6. Use cases — which metric for which study

The two outputs answer different physical questions. Pick by what the study’s driver acts on.

L_see — whole-array seeing level. Use when the effect acts on the PSF size / the whole focal plane at once and you want a single per-exposure loss number to regress against an external variable. It is artifact-free, uses no offset and no round-trip — the reliable choice for external-influence studies. - Mirror–air ΔT → mirror seeing → PSF broadening → loss (the immediate use case). - Dome seeing; ambient temperature; humidity / condensation effects on image quality. - Wind where the effect is image degradation (shake / turbulence broadening), not a coherent pointing shift. - Time / seasonal trends in seeing-driven throughput; sanity-checking the ETC’s delivered image quality. - Not for: the DAR field pattern or any pointing/offset-driven structure — that is L_field.

L_field — scale+shear DAR field pattern. Use when you care how the loss varies across the field from a coherent offset distortion, not the whole-array level. - Characterizing / monitoring the DAR compression pattern; validating the DAR model or PlateMaker against the delivered field. - Field-pattern anomaly searches — e.g. a rotator / hexapod / mount issue imprinting an unexpected low-order pattern. - Reliable for detection / relative / anomaly use at all airmass; the absolute magnitude is validated at am ≲ 1.6 and bracketed 11–16 μm at am > 1.8 (§3.3). Blind to per-fiber / high-order structure. - Not for: whole-array-level studies — that is L_see.

Neither — you need the dither. Per-fiber offsets, per-petal patterns, or any high-order (beyond affine) structure are below the edge-GFA affine scope (§2). Those require the dither offset field (sparse, special sequences), which is also the calibrator / ground truth for L_see and L_field (§4).

Rule of thumb: whole-array level → L_see; low-order field pattern → L_field; per-fiber / high-order → dither.

## 7. Using the metrics — the Exposure API

Both metrics are exposed as cached properties on telemetry_mining.Exposure, so the common case is a one-liner:

from telemetry_mining import Exposure
exp = Exposure(255020)
exp.L_see      # -> float: whole-array seeing loss level, or None
exp.L_field    # -> float: DAR field-distortion loss, or None

What you get. Each returns a single float per exposure, or None when the metric can’t be produced for that exposure (see Coverage below) — the module’s standard “None means unavailable” contract, so you can skip Nones rather than guard against exceptions.

How the value is produced — compute-direct, then fall back. Each property first tries to compute the metric directly from the offline inputs; if those aren’t available at this site (or desimodel isn’t installed) it falls back to a precomputed fiber_loss_metrics table if one is registered; otherwise it returns None:

direct compute → registered fiber_loss_metrics table → None

This is what lets the same attribute work at NERSC (direct) and at KPNO (table). You don’t choose the path — the property does. The two paths return identical values (the precompute script and the live attribute share one implementation, telemetry_mining.fiber_loss).

What each needs (direct path):

| metric | inputs | needs desimodel |
| --- | --- | --- |
| L_see | offline GFA seeing (FWHM_ASEC) | yes (FastFiberAcceptance + platescale) |
| L_field | offline GFA seeing + ETC per-frame guide drift + exp.parallactic | yes |

At KPNO the offline GFA reduction isn’t available, so the direct path can’t run — register the precomputed table (below) to serve both metrics there. exp.parallactic (degrees) is exposed for L_field’s zenith-frame rotation and is useful on its own.

Coverage — when you get None:

- L_see: available for any exposure with a valid GFA seeing measurement.

- L_field: needs a multi-frame exposure (there must be an intra-exposure drift to measure) — None for too-short / calibration exposures. It is reliable for detection / relative use at all airmass; its absolute magnitude is approximate above am ≈ 1.8 (§3.3).

Bulk use (e.g. the mirror-ΔT L_see study, or the |Δairmass| L_field study):

import pandas as pd
from telemetry_mining import Exposure

rows = [(e, Exposure(e).L_see, Exposure(e).L_field) for e in expids]
df = pd.DataFrame(rows, columns=["EXPID", "L_see", "L_field"]).dropna(subset=["L_see"])

Registering the precomputed table (for KPNO, or just to avoid recomputing): build it once with scripts/build_fiber_loss_metrics.py, then register it process-wide so every Exposure() sees it:

from telemetry_mining.tables import TableSource, DEFAULT_TABLE_SOURCES
DEFAULT_TABLE_SOURCES.append(
    TableSource("fiber_loss_metrics", path="data/fiber_loss_metrics.parquet", index_column="EXPID")
)
# exp.L_see / exp.L_field now fall back to this table wherever direct compute can't run

Lower-level access. The metric math itself lives in telemetry_mining.fiber_loss — l_see_from_fwhm, sheardrift_from_thru, l_field_from_drift — if you want to compute the metrics outside of an Exposure (e.g. in a batch job over precomputed inputs). That is exactly what build_fiber_loss_metrics.py does.

## 8. Worked examples

Runnable, heavily-commented notebooks under notebooks/ — each is a template for a common study pattern, and the notebooks (not this section) are the living source for the analysis detail. Run any of these under the DESI Master kernel — the direct compute needs desimodel, and its absence fails silently to None.

- mirror_seeing_lsee_example_selectexp.ipynb — L_see vs mirror−air ΔT (the mirror-seeing hypothesis). Demonstrates exp.L_see pulled in bulk with select_exposures (db_row specs for the free DB columns, a callable for the metric), and the asymmetric warm-mirror rise.

- lfield_dam_example_selectexp.ipynb — L_field vs intra-exposure |Δairmass| (a DAR scheduling lever). Demonstrates exp.L_field via select_exposures with max_workers (justified here — L_field does a real per-exposure ETC read) and a self-regenerating disk cache (rebuilt live when missing or built for a different sample), plus an airmass-controlled analysis and the derotated per-fiber quadrupole map behind the scalar.

- rcalibfrac_vs_lfield_example.ipynb — why RCALIBFRAC isn’t a clean metric: its per-fiber field is dominated by the rotating dipole artifact (D_rot/Q_rot ≈ 1.9) versus L_field’s pure quadrupole (D_rot = 0). A side-by-side, team-presentation comparison (runs locally on precomputed fields).