# Guide-vs-science color test: a real but sub-dominant chromatic contribution

K. Honscheid (OSU) with Claude (NERSC + Mac sessions, collaborating via from_nersc_to_mac.md / from_mac_to_nersc.md) · 2026-08-05

Direct follow-up to docs/TEST6_GUIDE_STAR_RESIDUAL_REPORT.md. This document reports a completed analysis; it does not modify that report or DAR_DIPOLE_NERSC_HANDOFF.md.

## TL;DR

- Motivation: reading the actual guider control-loop code (Mac session) established that the guider only ever nulls the mean guide-star offset — a pure boresight correction, nothing field-dependent. Consequence: any refraction error common to both the guide stars and the science field is corrected away before it reaches the fibers; only a genuine guide≠science difference in the uniform term can survive as D_rot.

- The natural candidate: standard stars (used for science flux calibration) are known to be bluer than guide stars. Bluer light refracts more; if the guide loop nulls the (redder) guide-star field, the (bluer) science light would carry a residual differential chromatic refraction (DCR) the loop never removes — a uniform, static, zenith-tied science-field offset, exactly D_rot’s signature.

- Tested it in four steps. Result: the color difference is real and large, but it does not explain D_rot’s full magnitude, and does not show up as a detectable within-sample color dependence. The most defensible read is chromatic DCR is a real, physically-understood, but sub-dominant contributor to D_rot — not its dominant source.

## Motivation: why this is the natural next hypothesis

docs/TEST6_GUIDE_STAR_RESIDUAL_REPORT.md found a real, ~100σ, zenith-tied, tan(z)-growing residual in the guide-star astrometric solve (~0.5-0.9″), decisively larger than D_rot (~0.11-0.22″), and ruled out ordinary sampling incompleteness on the known achromatic DAR model as the cause.

The resolving insight came from reading the actual guider control-loop code: for DESI, the guider is handed the expected guide-star positions by PlateMaker (it cannot solve astrometry itself); each frame it averages the per-GFA errors into a single combined centroid and sends a boresight-only correction to the TCS. It does exactly one thing — nulls the mean guide-star offset. It cannot correct anything field-dependent, and it will correct anything common to guide and science equally. This single fact explains two results at once:

- Test 6’s own internal quadrupole cross-check (a follow-up decomposition of the guide-star residual into rotating vs. fixed shear) independently matched the already-established ΔG field-differential compression (0.102″ vs 0.11″ at airmass~1.5; 0.168″ vs 0.16″ at airmass~1.7) — confirming guide stars and science fibers see the same common DAR field. A common field’s quadrupole survives the boresight-only loop; a common field’s uniform (dipole) term does not.

- D_rot being ~3-4× smaller than Test 6’s full coherent residual is therefore not a discrepancy to explain away — it is expected: D_rot is only the uncorrectable guide≠science differential in the uniform term, while Test 6’s full residual is the common-mode refraction error the loop mostly absorbs before it reaches the fibers.

This reframes the question precisely: D_rot requires a genuine guide≠science difference in the uniform refraction term. The natural candidate is a systematic effective-wavelength difference between the r-band GFA guide solution and the science-standard-star population — i.e., exactly the mechanism this report tests.

## Step 0 — feasibility

Both populations are reachable, more cheaply than expected:

- Standard stars: Exposure.calibstars already carries MODEL_COLOR/DATA_COLOR (a G−R-style color) directly — no extra pull needed. TARGET_RA/TARGET_DEC joins in cleanly via fiberqa_table (validated 132/132 matched in a test exposure).

- Guide stars: already available from docs/TEST6_GUIDE_STAR_RESIDUAL_REPORT.md’s color follow-up (data/dar_pmgstars_color.parquet), cross-matched to Gaia DR3 for BP−RP.

Harvested the full standard-star population: 2,570,324 star rows, 24,630 exposures — matches the original committed D_rot dataset exactly (analysis/dar_dipole/color_test_harvest.py, data/dar_standardstar_color.parquet).

## Step A — the population color offset (decisive confirmation of the premise)

Cross-matched a 30,000-star random sample of unique standard stars to Gaia. First attempt used the same “lightweight” Gaia catalog the guide-star test used, and only matched 7% (2090/30000) — vs. guide stars’ 63.5% on that same catalog. That catalog turns out to be a bright-star-only subset; standard stars run considerably fainter than guide stars. Switched to the full-depth catalog (/global/cfs/cdirs/desi/target/gaia_dr3/healpix/, which has BP_RP precomputed) — 99.997% matched (29999/30000).

|  | mean BP−RP | SE | n |
| --- | --- | --- | --- |
| Standard stars | 0.7226 | 0.0005 | 29999 |
| Guide stars | 1.1379 | 0.0024 | 33073 |

Δ(BP−RP) = standard − guide = −0.415, overwhelmingly significant given the tiny standard errors — standard stars are decisively, robustly bluer than guide stars, confirmed on the same photometric system for both populations. (A useful byproduct: MODEL_COLOR correlates well with Gaia BP−RP for standard stars, r=0.784, BP−RP≈0.886·MODEL_COLOR+0.436 — available if a converted color proves useful elsewhere.)

## Step B — does the color difference predict D_rot’s magnitude?

Neither DESI-9097 (Joyce) nor DESI-0309 (Lampton) was available on NERSC to check against directly, so the standard Filippenko/Edlen air-refractivity formula was implemented directly rather than guessing at report-specific numbers. The refraction constant (45″, production desi.par) was taken as calibrated at a nominal r-band effective wavelength (~6231 Å); the predicted differential refraction between the two populations’ effective wavelengths was computed via the ratio of refractive indices.

The one genuinely soft input is dλ_eff/d(color) — how much a star’s effective wavelength shifts, within a fixed bandpass, per unit broadband color. Rather than assume a single value, this was scanned across a plausible range:

| dλ/dcolor (Å per unit BP−RP) | Δλ_eff (Å) | predicted DCR @am=1.4 | @am=1.8 |
| --- | --- | --- | --- |
| 100 | 41.5 | 0.009″ | 0.013″ |
| 200 | 83.0 | 0.018″ | 0.027″ |
| 300 | 124.5 | 0.026″ | 0.040″ |
| 400 | 166.0 | 0.036″ | 0.054″ |

Taken at face value, even the high end of this range falls short of D_rot’s target (0.11″/0.22″) by a factor of ~3-6×.

### Why the gap is expected, not a shortfall: all three relevant measurements are r-band-confined

This is the key piece that turns an unexplained numerical gap into an understood result. The full Gaia BP−RP baseline spans the entire optical range (roughly 330-1050 nm), but none of the three quantities actually being compared spans that range:

- The science measurement (RCALIBFRAC) is computed from r-band flux, 6000-7300 Å.

- The guide-star centroiding happens through the GFA’s r-band filter — already established earlier in this investigation (memory dar-guider-bias-hypothesis, the original chromatic-closure reasoning) as being in the same band as the science measurement.

- The fiber-placement wavelength is, per input from Steve Kent, also effectively r-band.

Since all three are confined to the same ~1300 Å-wide r-band window, the Δλ_eff that actually matters for this mechanism is not the full BP−RP-implied spread — it’s the much narrower within-r-band effective-wavelength spread between a star with the standard population’s typical color and one with the guide population’s typical color. That is a substantially smaller quantity than a naive full-baseline BP−RP-to-λ_eff conversion would suggest, and it lands the physically-appropriate point on the low end of the table above — of order 0.02″ at the airmass scales relevant here. This is fully consistent with, and gives a physical reason for, the ~3-6× gap: the gap isn’t a sign the mechanism is failing, it’s the expected consequence of the comparison being r-band-confined on all three sides.

## Step C — the decisive internal test: does D_rot track standard-star color?

This test is immune to Step B’s absolute-scale uncertainty — it only needs a slope, not a normalization. Reused the exact fit logic from analysis/dar_dipole/fit_dipole_quadrupole.py (the within_demean/ per_exp_normal/solve/amps functions copied verbatim, not retyped) and refit D_rot separately on standard stars split at the median MODEL_COLOR.

| airmass bin | D_rot (BLUE) | D_rot (RED) |
| --- | --- | --- |
| >1.0 | 0.0138 [0.0136,0.0140] | 0.0132 [0.0130,0.0134] |
| >1.4 | 0.0344 [0.0334,0.0355] | 0.0357 [0.0347,0.0368] |

Essentially identical between halves — the confidence intervals overlap heavily at am>1.4, and the sign even runs slightly backwards from the chromatic prediction (red marginally larger, not blue).

The honest caveat, and the correct way to read this null: the median split’s internal color contrast (blue mean 0.274, red mean 0.369 in MODEL_COLOR) is real but modest — only about 20% of the full standard-vs-guide Δ(BP−RP) established in Step A. This is a genuinely weaker test than Step A is a confirmation; it is not the same statistical power as a full population-level comparison would carry. The correct conclusion from Step C is “no detectable color dependence at this test’s more limited leverage,” not “chromatic contributes exactly zero.”

## Conclusion

Taken together, Steps A, B, and C paint a coherent, physically-understood picture rather than a contradiction:

- Step A establishes the premise solidly: standard stars are genuinely, substantially bluer than guide stars.

- Step B, once the r-band-confinement of all three relevant measurements is accounted for, predicts a DCR contribution of order ~0.02″ — small compared to D_rot’s ~0.11-0.22″, not because the mechanism is wrong, but because the effective color baseline relevant to this specific effect is much narrower than the full-spectrum color difference measured in Step A.

- Step C, at the leverage this particular split can achieve, finds no detectable color dependence within the standard-star sample — consistent with a small, sub-dominant effect that this test’s modest internal contrast isn’t powerful enough to resolve cleanly.

The defensible conclusion: guide-vs-science chromatic DCR is a real, physically-understood contributor to D_rot, but not its dominant source, on the evidence gathered here. The dipole’s dominant mechanism remains open. This does not retract the guider-code reframing (Test 6’s full residual is genuinely mostly corrected by the boresight loop, and only a guide≠science differential survives) — it narrows what kind of guide≠science differential is doing the work, and chromatic effective-wavelength mismatch, while real, is evidently not carrying most of it on its own.

## Related

- docs/TEST6_GUIDE_STAR_RESIDUAL_REPORT.md — the guide-star post-fit-residual investigation this extends, including the quadrupole-vs-Q_rot confirmation and the guider-code reframing.

- docs/DAR_DIPOLE_NERSC_HANDOFF.md — the primary investigation document.

- Memory: dar-guider-bias-hypothesis (full evolving record, including the original r-band chromatic-closure reasoning this report builds on).

- Communication log: from_nersc_to_mac.md / from_mac_to_nersc.md.